Configuration file - BPQCFG.TXT

PORTS Section.

This section of the configuration file defines the links from the switch

to the outside world. These will normally be radio links, but could also

be a link to another computer running another copy of the switch, or similar

software (eg NOS).

Please note that there is no relationship between the 'Virtual' COM ports

defined in the TNCPORT section, which are used by the BBS or other program

running abouve the switch, and these external links. You may use your

'real' COM1 port to connect to a KISS TNC, even if you have defined a

'virtual' COM1 in the TNCPORT section.

There are two basic classes of links supported by the switch. One uses

normal ASYNC ports to communicate with TNCs running in KISS mode, and the

other uses special HDLC cards which fit inside the PC, and interface to

the radio via either an inbuilt or external  modem. Those currently supported

are the PacCom PC120, DRSI PC-PA, Thor RLC100, Baycom USCC-Karte and

OptoPcSCC.

Interrupt levels may be shared by any cards with suitable hardware.

Examples are all but the earliest DRSI cards, and a MS400 ASYNC card

modified as described by AA4RE in his MBBIOS documentation. However

there is a significant overhead in using the shared interrupt feature

- try to use it on relatively low speed links, and watch out for TX

underruns/RX overuns.

Each port is defined by a set of statements of the form:

PORT

PARAM=VALUE

..

..

ENDPORT

Samples of various configurations are given later, but the full list

of PARAM values supported is given below

PORTNUM=n

Number for use in PORTS display, connects, etc. Default is next

sequential number.

ID=text string *

Up to 30 chars, appears on PORTS display

TYPE=ASYNC/DRSI/PC120/RLC100/QUAD/INTERNAL/EXTERNAL/BAYCOM/PA0HZP *

Hardware type.

ASYNC is for a normal ASYNC card, or a shared interrupt card

such as the MS400

PC120, DRSI, RLC100, BAYCOM, PA0HZP are the internal HDLC cards,

the first two support up to two channels, the others up to 4.

QUAD is the Quadram Quadport, which allows up to 5 async links.

This version only supports a single Quadport. See later section

for setup details.

INTERNAL defines a 'loopback' port - anything sent on it will be

received. Intended to allow KISS applications (eg TCP/IP) to talk

to the switch.

EXTERNAL defines a user-supplied hardware driver.

PROTOCOL=HDLC/KISS/NETROM *

Protocol to be used on the link.

NETROM is to connect to the async port of a real NETROM

KISS is to connect to a KISS TNC

IOADDR=xxxH *

The base I/O address for the card. In hex, H is needed

INTLEVEL=n *

The interrupt level to be used. Any spare level may be used, up to

7 on an 8 bit machine, or 15 on a 16 bit machine. More than one card

may share a level if the hardware has the necessary logic. Also used

to define the software interrupt for EXTERNAL drivers.

SPEED=nnnn *

The radio speed for HDLC cards, or the async link speed for KISS

and NETROM links. If set to zero, the modem must supply a 1x clock.

(TXC on RTXC pin, RXC on TRXC pin.)

CHANNEL=A/B/..*

Selects port on HDLC cards, dual port KISS (eg KPC4) or Multidropped KISS

BBSFLAG=NOBBS

Stops direct connects to the BBS on this port.

QUALITY=nnn *

Default quality for nodes heard on this port. Set to zero to suppress

all level 3/4 activity (including NODES broadcasts) on this port.

MAXFRAME=n *

Maximum outstanding frames.

TXDELAY=nnn *

TX Keyup delay in milliseconds

SLOTTIME=100 *

CSMA interval timer (milliseconds)

PERSIST=nnn *

'Probability to transmit' value (1-255)

FULLDUP=0/1

Passed on to TNC if using KISS. Used to allow transmission whilst DCD

is active on HDLC cards, and to allow connection of a NETROM cluster

via a diode matrix.

SOFTDCD=0/1

If set to 1, system uses the detection of a valid frame to indicate that

the channel is busy, rather than the DCD signal from the modem. Used

only for HDLC cards.

TXTAIL=nnnn

TXTAIL timer in ms. Used only by KISS TNCs. If running on HF at 300 baud,

the default is probably too short. If others have trouble copying your

frames, try a value of 50 - 100.

FRACK=nnnn *

Level 2 timout in milliseconds

RESPTIME=nnnn *

Level 2 delayed ack timer in milliseconds

RETRIES=n *

Level 2 maximum retry value

PACLEN=nnn *

Default maximum packet length for this port

CWID=CALL

Only used on HDLC cards. CWID is sent every 29 mins.

CWIDTYPE=ONOFF

Send CWID using ON-OFF Keying. Without this param, the CWID will be sent

using normal modem tones. This is fine for low speed links, but faster

modems (eg the G3RUH) include scramblers which make it impossible to read

the CW. Note that the TX must have a pretty fast keying characteristic

to work with ON-OFF keying - certainly < 20mS.

PORTCALL=CALL

PORTALIAS=ALIAS

PORTALIAS2=ALIAS2

Additional callsign/alias for level 2 connects to the node for this port.

Should not be the same as BBSCALL

ALIAS_IS_BBS=0/1

If set then PORTCALL and PORTALIAS access the BBS instead of the node.

VALIDCALLS=CALLS

Up to about 40 callsigns, separated by single commas. If present, only

stations in the list will be allowed to connect to you on this port.

The SSID should not be specified, and multiple VALIDCALLS lines may be

included. End all but the last line with commas. The total length of the

list, including commas, is 256 bytes.

MINQUAL=NNN

Defines the minimum quality for a destination to be included in a

'Nodes' broadcast on this port.

QUALADJUST=NNN

Percentage to reduce the quality of a NODE in the NODES broadcast, if

the best neighbour is on the same port as the broadcast. Designed to

improve throughput if more than one channel is available, by discouraging

routes which cause a node to send a packet on the same port that it

received it on.

DIGIFLAG=0/1/255

Digipeat control flag for this port. 0 = dont digi, 1 = digi everything,

255 = digi UI frames only

DIGIPORT=N

Port to send digied frames on. Zero means the port it was received on,

any other number means send on that port. This is intended mainly to

allow a BBS on another machine, connected via a wire link, to send out

'MAIL FOR' messages.

MAXDIGIS=N

Limits the number of digipeaters that can be used to access this port

USERS=N

This limits the number of L2 sessions to the supplied number. Zero means

no limit. The limit applies to all L2 sessions (even those with other

nodes), but is only checked for incoming connects (ie you can make

outgoing connects even when the limit is reached). Intended mainly to

control the number of users on busy user access ports (especially 2m).

UNPROTO=

Unproto destination and digi string for this port. Use commas between

calls, and dont include the VIA.

L3ONLY=0/1

If set, will stop users (but not HOST sessions) issuing downlink

connects on this port.

INTERLOCK=n

Allows groups of ports to be defined which will not transmit at the same

time. A value of zero indicates no locking, if any other value is specifed

the code will make sure that no two ports with the same value can transmit

at the same time. Only valid on HDLC cards.

TXPORT=n

Allows frames for this port to be transmitted on a different port.

This is for use when you are operating a central node, with a single

transmitter, and a number of receivers.

MHEARD=Y/N

Enables or disables the MHEARD function on this port. There is a slight

overhead in maintaining the MHEARD list, so if it is not needed (eg on

an internode port) it may be disabled. Default is enabled.

KISSOPTIONS=POLLED,CHECKSUM,ACKMODE,SLAVE

These select my extended KISS mode.

These extended KISS options are supported by my BPQKISS EPROM for

the TNC2 and clones, and by special EPROMS for the KPC2, KPC4 and

KAM, available from Kantronics.

POLLED selects multi-dropped mode, where the TNC only

transmits when requested (Polled) by the PC. This allows

several TNCs to share an async port without any risk of

collisions.

CHECKSUM selects a mode where a checksum byte is appended

to each data message, and checked by the other end. This

reduces the risk of data corruption if errors occur on the

link from PC to TNCs. It may be used on its own, but should

always be used if POLLED mode is selected.

ACKMODE enables the transmission of 'ACK required' frames.

These cause the TNC to send a reply when the frame has been

transmitted. This will improve link performance by avoiding

the possibility of a retry being sent before the original

frame has even left the TNC. This mode should always be used

if the TNC support it.

SLAVE allows a PC to be multidropped along with one or more

KISS TNCs on a multidropped link. If you use SLAVE, you shouldn't

set POLLED, and you shouldn't use ACKMODE on either end.

Parameters marked with a * must be included, the rest are optional. The

validation is not good, and defaults are generally zero, so if you miss

out any of the essential ones, odd things may well happen!

Sample PORT Configurations:

1. 2 Port DRSI card set for IO addr = 300h, int level 5

PORT

ID=144.650 MHz 1200 Baud

TYPE=DRSI

PROTOCOL=HDLC

IOADDR=300H

INTLEVEL=5

SPEED=1200

CHANNEL=A

QUALITY=10

MAXFRAME=1

TXDELAY=500

SLOTTIME=100

PERSIST=64

FULLDUP=0

SOFTDCD=0

FRACK=7000

RESPTIME=2000

RETRIES=10

PACLEN=120

CWID=G8BPQ

ENDPORT

;

PORT

ID=432.675 MHz 1200 Baud

TYPE=DRSI

PROTOCOL=HDLC

IOADDR=300H

INTLEVEL=5

SPEED=1200

CHANNEL=B

BBSFLAG=NOBBS			; IF MY BBS LICENCE IS ONLY FOR 2M

QUALITY=10

MAXFRAME=1

TXDELAY=500

SLOTTIME=100

PERSIST=64

FULLDUP=0

SOFTDCD=0

FRACK=6000

RESPTIME=1500

RETRIES=10

PACLEN=120

CWID=G8BPQ

PORTCALL=G8BPQ

PORTALIAS=NOTTM

;

ENDPORT

2. Standard KISS TNC on primary ASYNC card. (COM1). To use the secondary

ASYNC card (COM2), change IOADDR to 2F8H, and the INTLEVEL to 3.

PORT

ID=144.650 Mhz 1200 Baud

TYPE=ASYNC

PROTOCOL=KISS

IOADDR=3F8H

INTLEVEL=4

SPEED=9600

CHANNEL=A

QUALITY=10

MAXFRAME=2

TXDELAY=500

SLOTTIME=100

PERSIST=64

FULLDUP=0

FRACK=7000

RESPTIME=2000

RETRIES=10

PACLEN=120

ENDPORT

The second port of a multiport TNC (Such as the KAM or KPC 4) is defined as

follows:

ID=432.675 Mhz 1200 baud

TYPE=ASYNC

PROTOCOL=KISS

IOADDR=3F8H

INTLEVEL=4

SPEED=2400

CHANNEL=B

QUALITY=10

MAXFRAME=2

TXDELAY=500

SLOTTIME=100

PERSIST=64

FULLDUP=0

FRACK=7000

RESPTIME=2000

RETRIES=10

PACLEN=120

QUALADJUST=5

ENDPORT

3. Quadram Quadport.

The Quadram Quadport card supports up to 5 (yes I know it says quadport!!)

async ports. This version only supports one card - if anyone needs more,

let me know. You do not need to define all ports, but you must start with

the first, and not leave any gaps.

The card should be configured with first port at 280h, and software

selectable interrupts (3 or 4 may be used)

The card will support KISS TNCS (either normal, or KPC4 type dual port), or

NETROM links (or a combination of both).

The following sample config file is for a Dual Port KPC4 on channel 1,

a normal KISS TNC on channel 2, and a NETROM async link on channel 3.

PORT

ID=KPC4 PORT 1

TYPE=QUAD

PROTOCOL=KISS

IOADDR=280H

INTLEVEL=3

SPEED=2400

CHANNEL=A

QUALITY=10

MAXFRAME=2

TXDELAY=500

SLOTTIME=100

PERSIST=64

FULLDUP=0

FRACK=7000

RESPTIME=2000

RETRIES=10

PACLEN=120

ENDPORT

PORT

ID=KPC4 PORT 2

TYPE=QUAD

PROTOCOL=KISS

IOADDR=280H

INTLEVEL=3

SPEED=2400

CHANNEL=B

QUALITY=10

MAXFRAME=2

TXDELAY=500

SLOTTIME=100

PERSIST=64

FULLDUP=0

FRACK=7000

RESPTIME=2000

RETRIES=10

PACLEN=120

ENDPORT

PORT

ID=NORMAL KISS

TYPE=QUAD

PROTOCOL=KISS

IOADDR=290H

INTLEVEL=3

SPEED=2400

CHANNEL=A

QUALITY=10

MAXFRAME=2

TXDELAY=500

SLOTTIME=100

PERSIST=64

FULLDUP=0

FRACK=7000

RESPTIME=2000

RETRIES=10

PACLEN=120

ENDPORT

PORT

ID=NETROM BACKEND

TYPE=QUAD

PROTOCOL=NETROM

IOADDR=2A0H

INTLEVEL=3

SPEED=2400

CHANNEL=A

QUALITY=10

MAXFRAME=2

TXDELAY=500		; THESE AREN'T USED, BUT I SUGGEST

SLOTTIME=100		; YOU PUT THEM IN IN CASE ONE DAY

PERSIST=64		; THEY ARE!

FULLDUP=1		; THIS IS NOW NEEDED FOR A SINGLE NETROM

; TNC. SET TO 0 IF USING A DIODE MATRIX.

SOFTDCD=0

FRACK=7000

RESPTIME=2000

RETRIES=10

PACLEN=120

ENDPORT

4. Baycom USCC.

This card has 4 ports, two running at 1200 (Ports A and B), one

TTL port (Port C), and one G3RUH compatible 9600 bps port (Port D).

The configuration is similar to the DRSI, except that port D should

have SPEED=0 (Not SPEED=9600).

John Wiseman, G8BPQ

24/1/2000