Baycom driver for BPQ Node Software

-----------------------------------

The BPQAX25 program is based on the AX.25 packet driver written by Pawel

Jalocha for use with NOS which I have modified to interface as an external

packet driver to the BPQ Node software.

BPQAX25 is a TSR program which is loaded before the BPQCODE program. It has

several command line options which are given below :

bpqax25 options: (default values in [])

?  prints this help message

-i <int_no>(hex) BPQ software interrupt number [60]

-I <irq>(hex)[p][s] COM IRQ number 2..7 [4]

-B <base>(hex)      COM base address 0..3ff [3f8]

-b <bit rate>(dec) [1200]

-c <carrier mode> possible choices are:

-cf = full duplex (do not care about channel busy - just transmit)

-cc = sense DCD modem line

-ct = sense data transitions [default] (hardware DCD)

-cd = deliver carrier signal from data analysis (software DCD)

-T <threshold>(dec)  software DCD threshold 0..100 [50]

-s <slot time>(dec) slot time in data bits [120]

-p <peristance>(dec) persistance/255 [64]

-h <tx head>(dec)  Transmitter head in data bit units [240]

-t <tx tail>(dec)  transmitter tail in data bit units [24]

-S  sound effects (beeps for every received frame)

Running the BPQAX25 program without any command line parameters will default

to using the modem on COM 1 at 1200 baud using a BPQ interupt of 0x60 (hex)

which is 96 decimal (used in BPQCFG.TXT - see below ).

To use COM 2, run the program as : BPQAX25 -I3 -B2f8

To configure it into BPQ, you must define a PORT in the BPQCFG.TXT file

setting TYPE to EXTERNAL and INTLEVEL to the value used in the -i option above

eg,

PORT

ID=144.650 Mhz on Baycom Modem

TYPE=EXTERNAL

PROTOCOL=KISS

IOADDR=2F8H

INTLEVEL=96     ; Note - this is decimal, -i above given in hex

SPEED=9600

CHANNEL=A

QUALITY=10

MAXFRAME=2

TXDELAY=500

SLOTTIME=100

PERSIST=50

FULLDUP=0

FRACK=7000

RESPTIME=2000

RETRIES=10

PACLEN=120

CWID=G7JJF

DIGIFLAG=1

DIGIPORT=0

USERS=0

UNPROTO=MAIL,NORMAN

MHEARD=N

ENDPORT

I am sure some of these parameters don't need to be specified, ie SPEED,

IOADDR etc but it doesn't cause any harm to leave them in.

The program only took a quick couple of hours to bodge together and it seems

to work ok with my setup. It is very unlikely that there are bugs in there

but if anything strange happens, let me know.

Please note that I cannot give support on the AX25 aspects of the driver

since I did not write it. You can find more information on this part of the

program plus fuller descriptions of the various command line parameters by

referring to the AX25.DOC file which Pawel distributes with his version of

the program. I did not include it here to save space.

If there is enough interest shown in this program, I may develope it further.

It is quite big at the moment due to parts of it being written in C but I

could rewrite it all in assembler (ie pretend to be a 'Real Programmer') if

there was a demand.

If you have any comments or bugs reports etc, you can contact me at the

following places :

Jon Welch,

50 Quarrydale Road,

Sutton In Ashfield,

Notts. NG17 4DR.

Great Britain

18th May, 1996

���������������������������������������������������������������������������ͻ

� G7JJF @ GB7MAM.#23.GBR.EU      AX25 �       Providing Support For :       �

� jon@g7jjf.demon.co.uk      Internet � TNCV142, MUTNC205, MUBAY102, WINAX25�

���������������������������������������������������������������������������͹

�           Latest Versions Of TNC Driver Softare Available From :          �

�                       http://members.aol.com/g7jjf/                       �

���������������������������������������������������������������������������ͼ