Packet monitor program

This program is designed to monitor packet activity under the BPQ Node

software and Desqview.

There are two files supplied;

MONITOR.EXE		The program.

MONITOR.NEW		A list of updates.

MONITOR.DOC		This file.

The program requires some command line parameters; the COM's port number

and the port number (which port(s) to monitor).

For example;

MH -p 3 255

| | |

| | |

| | |

| | |

| | \

| |  Monitor all Ports

| \

|  COM 03:

\

MCOM ON (optional)

The '-p' parameter is used to tell the program to monitor all packets (MCOM

ON) not just information packets. If the node can hear a lot of traffic, this

can cause corruption as the program will not be able to remove the data from

its 'TNC' (virtual) fast enough. To cure this, omit the '-p' and the program

will only monitor information frames (there may still be some corruption if

its very busy!).

The COM port number is the virtual BPQ COM port you allocate to this program.

The port number is the decimal representation of the binary number sent to the

BPQ code to tell it which ports to monitor. To monitor a port the ports bit

must be set. So to monitor port 1 only then type in a 1, port 2 a 2, port 3 a

4 and port 8 an 8. To monitor all ports type in 255 (sets all bits) and for

any other combination, the decimal number that sets the required bits.

To end the program use the 'Esc' key THIS WILL TAKE A FEW SECONDS (in DESQview

you may have to try twice). If you just close the DESQview window the BPQ node

will still think the program there and will still try to send the monitored

packets to the program.

I hope that all that was not too garbled! If you have any problems, contact

myself:-

Paul G4YFE @ GB7VRB

Paul Hounslow

16 West Mansion

Heene Rd

Worthing

West Sussex

BN11 3NT

0903 212301

or

Geoff G8DHE @ GB7VRB

Geoff Mather

72 Cranleigh Rd.,

Worthing

West Sussex

BN14 7QW

0903 32161

73's and good luck.