BPQNodeX

Ver 1.0

by Martin Schick KA4IWG

1996

Program:

BPQNodeX was written in Turbo Pascal and the source code is included.  It utilizes TPU’s from TurboPower Software which allow for the development of TSR’s and ISR’s.  Most of the procedural names for these units are self evident in their purpose.

BPQNodeX sits between the BPQCODE and PacketCluster routines and scans incoming packets to the cluster.  If any PC11 packet (DX spot) is received with the originating node callsign given BPQNodeX at start up, the packet is dropped and not passed to Packet Cluster for processing.  The node callsign can be up to 17 characters long, including any SSID of the call.  Thus, this routine can be used to block any DX spots from a designated node.  These spots will NOT be passed on to the next Packet Cluster node in the local system

This routine was written for use in PacketCluster systems in which external nodes, at some distance from the local cluster, may wish to connect to the local cluster and receive DX spots and mail, but the spots coming from this external network are not pertinent to the local system.  This routine will allow the one way passing of DX spots and the two way passing of all other information.  Some routines that provide a blocking function act by accessing a listing of all nodes NOT to block and blocking any that are not in the list.  This routine acts in reverse by blocking ONLY that one which is on the command line of the routine.

Usage:

In order to properly install BPQNodeX, it must be loaded from the same directory in which the BPQCFG.BIN  file, the configuration file for the BPQ code, resides.  This is due to the fact that BPQNodeX must examine this file to determine which interrupt BPQCODE is using at the present time.

BPQCODE must be installed prior to installing BPQNodeX.  Since BPQNodeX redirects the interrupt vector for the given interrupt of BPQCODE to BPQNodeX, the vector must have already been installed in order to redirect it.  Only the version 4 series of BPQ are acceptable to BPQNodeX since it utilizes the method of determining if BPQ is present introduced in the version 4 series.

BPQNodeX should not be installed from within the PacketCluster program by use of the DOS shelling routine.  Since the PacketCluster program usually enters the DOS shell by swapping memory and may still issue interrupts to BPQ while shelled, installing BPQNodeX at that stage will give unpredictable results.  However, the hotkey combination, mentioned later, can be used from within a DOS shell.

From my experience with usage on a PacketCluster node, it has been best to completely reboot the computer before loading BPQNodeX.  When I have tried to shutdown PacketCluster and either load or unload the routine, the computer has sometimes crashed.  I suspect this is due to trying to swap interrupt vectors while they are still in use by BPQ.  I have never had a problem when installing the routine from a cold start with the routine’s command line in a batch startup file.

When BPQNodeX is initially installed, the scanning process is active.  The scanning can be deactivated by use of the “<ctrl><X>“ key combination.  In a PacketCluster system, this can be done by simply shelling to DOS from a running PacketCluster routine and then issuing the “<ctrl><X>“ combination.

The initial version of BPQNodeX has been tested only under a DOS based 80386 system with the QEMM memory manager.  There has been no evaluation of the routine under Desqview or Windows, so no information can be given to its reliability under these platforms.

Options:

BPQNodeX is installed by entering “BPQNODEX [OPTIONS] <enter>“.  The options may be “/?” or “-?’, “?U” or “-U”, or the desired node callsign preceded by “/$” or “-$”.  Only one option is allowed on the command line, since only one is necessary.  Also, like DOS commands, there must be a space between the program name and the options when using the “-” key, but the space is not necessary if the “/” key is used.  The “/?” or “-?” option will bring up a short description of the various options available.  The “/U” or “-U” option can be used to remove a copy of BPQNodeX from RAM memory.

To enter the desired node callsign, the “-$” or “/$” option is used and the “$” is followed immediately by the callsign of the node to block, including any SSID.  For example, “BPQNODEX /$KA4IWG-4” would install the routine so that any incoming packet with DX information from the node KA4IWG-4 would be blocked out. This version of the routine will only accept a single node callsign.

There is only one hotkey combination which can be used in BPQNodeX.  This is the “<ctrl><X>“ combination.  DOS must be in control of the system when this key combination is used.  Therefore, by shelling out to DOS using the “DOS” command, this hotkey combination can be utilized.

The “<ctrl><X>“ combination will toggle the blocking process between active and inactive.  WHEN BPQNODEX IS FIRST INSTALLED, THE BLOCKING IS ACTIVE!  It can be deactivated by use of the “<ctrl><X>“ combination at a later time.  This is useful during such activities as contests during which time the extra spotting information from external clusters would put excessive burden on the cluster.  The blocking process can be reactivated by repeating the “<ctrl><X>“ combination.

Testing:

I have run this routine on my PacketCluster node for several days while connected to a node for which the spots were blocked.  All spots coming from that node were blocked as I watched in the monitoring mode.  Other nodes connected to the same source as I acted upon the spots.

I hope this routine will be helpful and would appreciate any suggestions or feedback on its usage.

Martin Schick  KA4IWG

BPQ, BPQCODE and BPQCFG are software routines written by John Wiseman, G8BPQ.  They are not public domain and may be used only by licensed radio amateurs.

PacketCluster is a trademark on Pavillion Software.

Desqview is a trademark of Quarterdeck Office Systems.

Windows is a trademark of Microsoft Corporation.

Turbo Pascal is a trademark of Borland International

The author of this software does not assume any liability for the use of the BPQNodeX routine and in no event will the author be liable for damages arising out of the use or inability to use this program.